Small Solar Cooker Team

Fall 2012

Final Report

Members:

Kait Hoffman

Colleen Lee

Table of Contents:

- Purpose

- Overview of Tests

- Oven Tests

- Materials Tests for Heat Basin

- Materials Results for Heat Basin

- Methods of Improvement

- Future Goals

- Purpose:

It is in the interest of the Solar Oven project team to design and test an oven for use in Ithaca, NY.  The latitudinal line along which Ithaca is located causes the sun to hit the earth at an angle of forty-two degrees in the winter.  As a result, the design of this cooker is different than that which will be taken to Nicaragua.  The design that is used by Las Mujeres Solares de Totogalpa in Nicaragua is a flat topped design.  This cooker, however, is used in Ithaca and has a glazing installed at forty-two degrees to allow maximum sunlight absorption during the winter.  Having an experimental small cooker readily available at Cornell aids in tests and raises awareness of how sustainable energy is helping improve people’s lifestyles.

Semester Goals:

The goal this semester for the Small Solar Cooker team was to test the cookers built during Fall 2011 and Spring 2012.  The focus for the semester was evaluating the performance of the ovens in various configurations.  The variables of interest included the two different ovens, the Fresnel lens, and materials used to build various heat basins.

At the beginning of the semester, the goal for the heat basin design has to determine the material that best retains heat, especially during the event when the source of energy is diminishing or unavailable. The original plan for the semester was as follows:

| Week of: | 10/8 | 10/15 | 10/22 | 10/29 | 11/5 | 11/12 | 11/19 | 11/26 | 12/3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing | F | N | F | B | BF | B | BF |  |  |
| Design of heat basin |  |  |  |  |  |  |  |  |  |
| Build of heat basin |  |  |  |  |  |  |  |  |  |
| Test of Heat basin |  |  |  |  |  |  |  |  |  |
| Final Report |  |  |  |  |  |  |  |  |  |

- Overview of Tests:

The plan for the semester was ambitious because we planned to get numerous tests completed each week. Testing of the heat basin in the two ovens has not been completed, but the testing of the two ovens with and without the Fresnel lens was completed. Additionally, the Small Solar Cooker sub team tested the effectiveness of four materials that act as the heat basin.  Our actual testing schedule was as follows:

| Week of: | 10/8 | 10/15 | 10/22 | 10/29 | 11/5 | 11/12 | 11/19 | 11/26 | 12/3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing | N | F | F,N |  |  | N | F |  |  |
| Heat basin research and materials order |  |  |  |  |  |  |  |  |  |
| Test of Heat basin |  |  |  |  |  |  |  |  |  |
| Data Analysis |  |  |  |  |  |  |  |  |  |
| Final Report |  |  |  |  |  |  |  |  |  |

Note: N denotes an oven test without the Fresnel lens, whereas F denotes an oven test with the Fresnel lens.

- Oven Tests:

The oven tests that were completed followed the same procedure—simply changing the oven and adding or subtracting the Fresnel lens from the oven set up.  Eight thermocouples were utilized during each test to measure temperatures at various locations: the ambient air, three locations within the oven, the water temperature in the canister in the oven, the material on the inside of the oven wall, and the material on the outside of the oven wall.  A pyranometer was used to measure the solar irradiance during the tests, allowing us to calculate the power for each test and comparison.  Each configuration was tested for two hours in the sunlight and two hours after in the shade.

Oven Results:

The following graphs display the temperature (C) vs. time plots from the oven configuration tests.  Below each graph is the power for the test, the time it took the water inside the can to reach 45 C, and the resulting power in kWh that it took to bring the water to 45 C.

Power: 0.3884 kW		Time to 45 C: 0.7444 hrs.		Power to 45 C: 0.2891 kWh

Power: 0.7801 kW		Time to 45 C: 1.6305 hrs.		Power to 45 C: 1.2719 kWh

Power: 0.2558 kW		Time to 45 C: 0.7417 hrs.		Power to 45C: 0.1897 kWh

Power: 0.5038 kW		Time to 45 C: 0.0500 hrs.	            Power to 45 C: 0.02519 kWh

Ideally, we would have used a temperature of 95 C instead of 45 C for our comparison, but because the highest temperature reached by the old oven water was only 45 C, our statistics for all tests are based on 45 C.  After analyzing our data, we determined the best oven configuration is the new oven with the lens because it required the least amount of power.

It was unexpected that the old oven with the lens required more power than the old oven without the lens, so our sub team plans to re-test these two configurations for more accurate data, or at least confirm the current conclusion on the performance of the old oven.

- Materials Tests for Heat Basin:

The material tests for the heat basin design were handled differently.  Our purpose for testing each material was to determine which material would serve as a better basin material.  We chose several materials to test, including black iron, black sand, pizza stone, and canola oil.  Each of these materials was tested in the indoor solar cooker apparatus for a period of one half hour under the lights and one hour after the lights were turned off.

- Materials Results for Heat Basin:

In the graphs, the data for the materials were normalized by the mass of its respective material (temp/mass), while the data for the air was not normalized (temp).  We have two sets of measurements to see the relationship between the temperature of the oven and the temperature of the heat basin material. Ultimately, we want to maximize the temperature of the small cooker ovens with the heat basin integrated; as a result, we looked at how hot the air in the oven got while testing our materials for the heat basin.  Then we looked at the material temperature; we looked for the hottest item, after the measurements were normalized by each material’s weight.  Finally, we considered the cool down period to determine which material kept the oven the hottest for the longest period of time.

Considering the three criteria, we determined the pizza stone to be the best for our uses. Additionally, we are considering an alternative heat basin set up, where we could combine the black iron and the pizza stone. We think this set up will give us desirable results also because black iron is more capable than pizza stone in retaining heat once the heat source is gone.

- Methods of Improvement:

After testing the ovens, we believe that the newer oven is performs better than the older oven.  In the future, We would like to analyze the air flow in the ovens using the data we have and the data we will collect in the future to determine ways to design a more effective oven.

The testing processes are fairly refined for the data we collected, but improvement in consistency of data would be a good goal.  It seems some of the oven tests have some spikes in the data that are unrealistic and eliminating these issues will be useful in the future.

- Future Work:

This semester we started the heat basin project and we plan to complete it next semester.  The design and testing of the basin will occur in the spring.  We would also like to design and build a new oven using some of the information we have acquired through testing this semester.  Lastly, if time permits, we would like to do more research and testing to determine the optimal placement of the lens. Although we have been directing the focal point of the lens to the back of the ovens, we would like to challenge that idea and see if there is a more efficient lens configuration.