Small Solar Cooker Team

Spring 2013

Final Report

Members:

Kait Hoffman

Colleen Lee

Nathan Ghabour

Table of Contents:

- Purpose

- Semester Goals

- Testing

- Results

- Future Goals

- Purpose:

It is in the interest of the Solar Oven project team to design and test an oven for use in Ithaca, NY.  Having the cooker around aids in tests and boosts awareness by being available for use and demonstration in local areas. The design of this cooker is different than that which will be used in Nicaragua.  The design that is used by Las Mujeres Solares de Totogalpa in Nicaragua has a flat top.  In order for the solar cooker that is used in Ithaca to optimally absorb the sunlight during the winter, the glazing has been installed at forty-two degrees. The reason for the placing the glazing on a forty-two degrees tilt is due to latitudinal line along which Ithaca is located, which causes the sun to hit the earth at an angle of forty-two degrees during the winter.

- Semester Goals:

The goal for this team was to test various glazing materials with the oven to determine an optimal configuration.  Our project required the Small Cooker Team to research and identify multiple clear glazing materials that were suitable for extreme temperatures in the oven.  The team used 3 different plastics: Reynolds oven bag, natural polypropylene lined smooth, and natural polypropylene stress relieved sheet. The two types of natural polypropylene are composed of the same materials; the only different is that the natural polypropylene lined smooth is .03 inches in thickness, whereas the natural polypropylene stress relieved sheet is .125 inches in thickness. Each of the 3 plastics were tested individually, and the we also tested the Reynolds oven bag with natural polypropylene lined smooth. We tested 5 different glazing configurations:

- 2 layers of Reynolds oven bag using 2 panes to separate them

- 1 layer of natural polypropylene lined smooth (single pane)

- 1 layer of natural polypropylene stress relieved sheet (single pane)

- the top layer was the Reynolds oven bag and the bottom layer was natural polypropylene lined smooth, using 2 panes

- the top layer was the natural polypropylene lined smooth and the bottom layer was the Reynolds oven bag, using 2 panes

The semester schedule after some planning time was as follows:

| Week of: | 3/4 | 3/11 | 3/18 | 3/25 | 4/1 | 4/8 | 4/15 | 4/22 | 4/29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Research of Materials |  |  |  |  |  |  |  |  |  |
| Preparation of Apparatus |  |  |  |  |  |  |  |  |  |
| Pre-Testing |  |  |  |  |  |  |  |  |  |
| Testing |  |  |  |  |  |  |  |  |  |
| Final Report |  |  |  |  |  |  |  |  |  |

- Testing:

The tests for the ovens were run with the same procedure, simply changing the glazing combination.  The apparatus used for testing included two shop lights mounted on top of each other vertically and tilted approximately forty-five degrees. Additionally, the glass for each shop light was removed so that the UV rays from the lights can reach the oven.  The oven was then placed a distance from the lights to allow for a pyranometer reading of approximately 0.850 kilowatts/meter2; this distance was marked on the floor to ensure uniformity among all the trials. Then, water-filled canister was placed in the oven, in the same spot; this spot for the canister was also marked inside the oven.  The mass of each canister of water was recorded.

To get a reading of the temperature of the water and the air inside the oven, thermocouples were placed in various locations throughout the oven.  Each test was 2 hours in total; during the first hour, the lights were turned on, and immediately after, the lights were turned off.  Data was recorded every second during those 2 hours.

|  | Average | Std. dev |
| --- | --- | --- |
| Max Temperature/Gram (Water) (°C/gram) | 0.273 | 0.0126 |
| Pyrometer reading (kwatt/m2) | 0.945 | 0.0335 |
| Final Temperature (Water) (°C/gram) | 0.216 | 0.00711 |

- Results:

Figure 1

Figure 2

Polypropylene Lined Smooth Plastic:

Our first new material was a thin sheet of Polyproplene measured at .03 inches. The test went fairly well in all three trials that we ran having a high Pyranometer reading each test.  Looking at water temperature one can see that the sheet worked really well heating up our water to .273° Celsius /gram very close to the oven bags we use at  .297°C/g. Both glazing materials also were able to maintain around the same temperature at the end of the experiment.  The only worry observation we saw was that the plastic deformed with each test and after testing the plastic stayed deformed.

Figure 3

Figure 5

Figure 4

Oven Bag with Polypropylene Plastic Outside:
This was the first test we ran that combined two different glazing materials. The oven bag and the plastic both worked quite well in our testing individually so we hoped that combing them would help insulate the oven better and keep our oven hotter. Looking at the max temperature, we saw a decrease in max temperature achieved. Our greatest surprise was the final temperature was significant decrease in final temperature. Having both materials somehow made the oven less insulated and let heat escape.

Figure 6

|  | Average | Std. dev |
| --- | --- | --- |
| Max Temperature/Gram (Water) (°C/gram) | 0.255 | 0.0448 |
| Pyrometer reading (kwatt/m2) | 0.879 | 0.0514 |
| Final Temperature (Water) (°C/gram) | 0.189 | 0.02169 |

Oven Bag with Natural Polypropylene Lined Smooth Inside:

This set-up is simply a reverse of the previous glazing configuration analyzed above; the same materials were used, but the placement of the materials was reversed. The oven bag was placed on the outer pane, and the natural polypropylene lined smooth was placed on the inner pane. The average maximum temperature and average maximum temperature was .228 °C/gram and .190 °C/gram, respectively. The average maximum temperature for this set-up was much smaller than that of the preceding set-up mentioned above. The average final temperature, however, was comparable between these 2 set-ups.

|  | Average | Std. dev |
| --- | --- | --- |
| Max Temperature/Gram (Water) (°C/gram) | .228 | .040 |
| Pyrometer reading (kwatt/m2) | .844 | .055 |
| Final Temperature (Water) (°C/gram) | .190 | .007 |

Trial #1:

Figure 7

Trial #2:

Figure 8

Trial #3:

Figure 9

Natural Polypropylene Stress Relieved Sheet:

Only 1 layer of natural polypropylene stress relieved sheet was used. The average maximum temperature and average maximum temperature was .226 °C/gram and .182°C/gram, respectively. The results for this set-up were least satisfying for both the average maximum temperature and average final temperature; however, it barely underperformed the set-up where the oven bag was on placed on the outside and the natural polypropylene lined smooth was on the inside.

|  | Average | Std. dev |
| --- | --- | --- |
| Max Temperature/Gram (Water) (°C/gram) | .226 | .005 |
| Pyrometer reading (kwatt/m2) | .890 | .015 |
| Final Temperature (Water) (°C/gram) | .182 | .045 |

Trial #1:

Figure 10

Trial #2:

Figure 11

Trial #3:

Figure 12

Oven Bag:

Two layers of Reynolds oven bag, separated by 1 pane, were used for this glazing configuration.  This set-up was most successful in two dimensions: it had the highest maximum temperature reached after 1 hour and it also had the highest final temperature after 1 hour of cooling off. The oven bag’s average maximum temperature and average maximum temperature was .297 °C/gram and .213 °C/gram, respectively. The other 4 glazing set-ups had average maximum temperatures ranging from .226 °C/gram and .273 °C/gram. The average final temperature for the other 4 set-ups ranged from .182 °C/gram to .216 °C/gram. In both dimensions, polypropylene lined smooth plastic performance did not measure up to that of the oven bag’s, but fared closely.

|  | Average | Std. dev |
| --- | --- | --- |
| Max Temperature/Gram (Water) (°C/gram) | .297 | .035 |
| Pyrometer reading (kwatt/m2) | .908 | .008 |
| Final Temperature (Water) (°C/gram) | .213 | .012 |

Trial #1:

Figure 13

Trial #2:

Figure 14

Trial #3:

Figure 15

- Future Work:

While running the tests on the different combinations of glazing, the Small Cooker team studied a limited number of characteristics to analyze the performance of the glazing materials: the temperature of the water during different stages of the tests. For subsequent semesters, the Small Cooker Team should examine other characteristics of the glazing, such as the resilience of the glazing materials and the amount of light that each configuration of glazing materials allows to pass through.

The natural polypropylene lined smooth (thickness of .03 inches) showed signs of deformation (such as minor warping) during the first hour of each trial, which is when the plastic was exposed to direct light. Similarly, during a separate testing, hot air accumulated between the double-paned Reynolds oven bag, causing the top layer of the oven bag to protrude and show signs of a stressed state. The Small Cooker Team, however, did not keep record of when the actions described above occurred, in relation to the first hour of each test. This type of information would be useful because we can compare the time in which each of the glazing materials exhibited the described physical changes to when it occurs for the Reynolds oven bags, which we concluded is the best glazing material configuration.

We also believe the Small Cooker Team should see how many times each glazing material can be used before there are noticeable physical changes in the plastics even when they are observed during their resting stages, which is when the plastics have not been exposed to direct light for at least several hours. This type of analysis is meaningful because it will reveal how short or long the useful life of each plastic is when they are used as glazing materials. If the glazing material needs to be replaced by new ones after a few number of usages, the individual using the solar cooker may find this inconvenient, and thus impractical, since the procedure of changing the glazing regularly is tedious.

The Small Cooker Team should also determine the proportion of light each glazing configuration allows to pass through. With this additional set of data, it would be interesting and useful to see if there is a direct correlation between the proportion of light passing through each glazing configuration and the maximum temperature of the water. In order to collect data for this set of tests, we need 2 pyranometers. One will be placed on top of the outer pane, as usual. The second pyranometer should be placed directly behind the layer(s) of glazing configuration. In order to prevent the pyranometer from overheating, the glazing materials and their wooded-pane should not be attached to the solar oven; a set-up in which the glazing materials and the wooded pane is freely-standing is sufficient in executing this test.

The Spring 2013 Small Cooker Team devoted the entire semester to testing different glazing configurations. The type of testing we did deviated from the works of the preceding sub team from Fall 2012. The sub team from Fall 2012 made progress in identifying a good heat basin material to use: a black-painted pizza stone. The conclusions, however, were based on the tests that were done using artificial lighting. The next step in advancing the heat basin study would be to examine the performance of heat basin inside the small cooker, in an outdoor setting.  We hypothesize that the solar oven will take longer to heat up in an outdoor setting as opposed to an indoor setting because the artificial lights used for the heat basin test is probably much stronger than the sunlight Ithaca gets on an average sunny day.