The trend that with increasing capture velocity, removal efficieny decreases is simlar to the result under the condition of 5 NTU and 500 NTU raw water turbidity. However, this kind of decrease is not that apparent after we use larger dose of alum.

In this picture we can see that there is a big jump of removal efficiency after using alum dose of 1.5mg/L and higher. In case of causing by the abnormal running of appatuas, we redo the experiment under alum dose of 0.5mg/L. However, the removal efficiency shown is similar to the first result. So we are considering that this jump of removal efficiency is due to larger alum dose. With raw water turbidity of 50NTU, alum dose may play a critical role in forming enough flocs that can settle down. Since for raw water turbidity as low as 5 NTU, although we increase alum dose, turbidity is still to0 low to form many flocs that can be removed. On the contrary, when raw water turbidity is as high as 500 NTU, the turbidity is large enough to offer enough collisions and combinations between particles. Thus alum dose is not such a criticle limit in forming enough flocs. However, for raw water turbidity of 50 NTU, dose of alum may be necessary for offering enough chances for particles to form flocs with appropriate size.