Echocardiography Summary

Three modes of echo: M-mode, two-dimensional (2-D, real-time), and Doppler

Physics:
Uses pulsed, high-frequency sound waves, which are reflected from body tissue interfaces back to the transducer

When the ultrasound beam meets an interface between differing biologic tissues, it is reflected, refracted and absorbed

Because the velocity of sound through tissue is known (1540m/sec), the distance from the transducer to the various reflective interfaces can be calculated and the resulting images displayed

Echo intensity

Depends on several factors

More reflective tissue boundaries (greater mismatch in acoustic impedance, which is related to tissue density) produce stronger echoes

Orienting the ultrasound beam perpendicular to the structure of interest produces a

stronger echo signal with 2D and M-mode

Beam energy decreases as the distance from the transducer increases because of beam

divergence and absorption, scatter and reflection

Very reflective interfaces prevent visualization of deeper, soft tissue interfaces

Transducer frequency and ultrasound power output and processing controls also

influence

Transducer frequency

Higher frequency ultrasound waves produce better resolution of small structures; however,

have less penetrating ability

Lower frequency provide greater depth of penetration but less well-defined images

Use highest frequency that allows for adequate penetration- 3.5MHz for large dogs to >7.5MHz for cats and small dogs

Descriptive terms

Echogenicitiy is related to tissues’ ability to reflect sound- strongly reflect=hyperechoic; poorly reflect=hypoechoic; don’t reflect=anechoic

Acoustic enhancement= tissue behind an area of sonolucency appears hyperechoic because more energy reaches it

Acoustic shadow=tissue beyond a strongly hyperechoic object shadowed

Echocardiographic exam:

Imaging: right and left parasternal transducer positions are used most often, probe placed over precordial impulse area, and position adjusted to find a good acoustic window

Artifacts:

Can result from misplacement of displayed echos returning from secondary vibrations, not along the primary ultrasound beam path, reverberation artifact from deeper structures, or from poor lateral resolution, when the structure is aligned parallel, not perpendicular to the beam

Inadequate skin-transducer contact, improper adjustment of u/s machine controls, a low frequency transducer and other equipment limitations

Patient factors- barrel-shaped, obesity

Lung artifact, rib interference

Systematic exam

Measurements made; assessment of cardiomegaly remains somewhat subjective, especially regarding right heart

Diastolic RV wall thickness should be <50% (and usally ~33%) of LV wall thickness

Measurements in cats- consistent;  dogs- variable

2-D Echo:

Standard 2D views:

Most of views are obtained from the right or left parasternal positions

Occasionally obtained from subxiphoid (subcostal) or thoracic inlet (suprasternal)

positions

Long-axis (sagittal) views are obtained with the imaging plane parallel to the long axis of the heart

Short-axis (transverse, horizontal) views are perpendicular to this plane

Progressive angulation of the transducer yields image planes from apex to base

Images are described by the location of the transducer and the imaging plane used

From the short-axis view, the transducer is rotated 90 degrees counterclockwise to

obtain the long-axis views

Heart base is positioned to the right of the screen and the apex to the left

Left apical 4 chamber view provides good visualization of the ventricular inflow tracts and chamber proportions

Left caudal (apical) transducer position:

4 chamber view (inflow) is obtained by orienting the ransducer index mark toward the animal’s left and slightly caudal

5 chamber view (outflow)

Two chamber view obtained by orienting the ultrasound beam plane parallel to the long axis of the heart, almost perpendicular with the long axis of the body

Rotation of the beam more cranidorsally-to-caudoventrally yields the LV outflow view

Left cranial transducer position:

Short-axis view is obtained by moving the transducer slightly more cranial on the chest wall (from the left apical position) and rotating the beam so it is almost perpendicular to the long axis of the body and heart)

Provides optimal orientation to the right ventricular inflow and outflow tracts

Long-axis view: Beam plane is rotated almost parallel to the long axis of the heart to align with the ascending aorta

More medial angulation allows the RV inflow tract, RA, Rau to be seen

Lateral angulation reveals the RV outflow tract and main PA

Left atrial size:

Best assessed using 2-D- compared to aortic root dimension

Methods:

Measure the cranial-caudal diameter (top to bottom) at end-systole using a right parasternal long-axis 4 chamber view

Cat: N<16mm

Because of great variation related to body size, the LA can be compared with the 2-D aortic root diameter measured across the sinuses of Valsalva in diastole

1.6-1.9 or ~1.6 (95% CI=1.1-2)

Contrast Echo:

Bubble study

Injection into a peripheral vein- normal right side

Bubbles on the left or in the aorta would indicate right-to-left shunt

M-Mode Echo:

Provides a one-D view of the heart; appropriate cursor placement is guided by 2-D

Provides a cleaner resolution of cardiac borders and more accurate timing of events within the

cardiac cycle b/c of high sampling rate

Standard M-mode views:

Obtained from right parasternal transducer position

Appropriate cursor placement performed using right parasternal short-axis view

Common measurements and normal values:

Systolic and diastolic thicknesses of the LV wall and IVS and LV chamber dimensions should be determined at the level of the chordae tendineae

Diastolic measurements are made at the onset of the QRS complex

Systolic measurements are made from the point of peak downward motion of

the septum

Septum and LV wall normally move toward each other in systole

Paradoxic septal motion- septum moves away from the LV wall and toward the transducer in systole- RV volume/pressure overload

Precludes accurate assessment of LV function using fractional shortening

LV function:

Fractional shortening= % change in LV dimension from diastole to systole

Assumes that the ventricle contracts uniformly and is limited by being dependent on ventricular loading conditions

HR and rhythm as well as regional wall motion abnormalities can also affect

Normal dogs: 25-40 or 45%; normal cats: >35% or 30-55%

Other indices of LV performance= mean velocity of circumferential fiber shortening (Vcf) and percentage change in IVS or LV wall thickness

Normal Vcf=1.6-2.8 circumferences/second in dogs; 1.3-4.5 circumferences/second in cats

=(LVIDd-LVIDs)/(LVIDd x LVET); LVET (LV ejection time) derived from the time between aortic valve opening and closing on M mode or the duration of aortic flow signal on Doppler exam

Normalized for HR= (LVIDd-LVIDs) x 100/(LVIDd x LVET x HR)

LV free wall thickening (%FWth)=( LVFWs-LVFWd )x 100/LVFWd

Use of the calculated end-systolic volume index (ESVI= ESV/BSA) has been suggested as a more accurate way to assess myocardial contractility in the presence of mitral regurg

Compares LV size after ejection with body size rather than with the volume-overloaded end-diastolic ventricular size

ESVI <30ml/m2 normal, 30-60ml/m2 mild LV systolic dysfunction, 60-90ml/m2 moderate dysfunction, >90ml/m2 severe dysfunction

Right parasternal long-axis view recommended

ESV derived from 2D:

Short axis area/long-axis length=5AL/6 (A=left ventricular area; L=left ventricular length)

Long-axis area length= 8A2/(3πL)

Modified Simpson’s Rule (method of disks)

Mitral valve E-point to septal separation (EPSS)= distance between

maximal mitral opening in early diastole (E point) and endocardial surface of interventricular septum

Mitral valve:

E point: Maximal opening of the valve during rapid ventricular filling

Normal animals E point close to IVS

Poor myocardial contractility is the most common cause of an increased

E point to septal separation (EPSS)

Normal EPSS is </=6mm in dogs, 4-5mm in cats

A point: Atrial contraction

With tachycardia, A point merges with E

Dynamic LV outflow obstruction causes the anterior mitral leaflet to be sucked

toward the septum during ejection (SAM)

Diastolic flutter of the anterior mitral valve leaflet caused by aortic valve regurg

Aortic root:

Diameter measured at end-diastole

Amplitude of posterior to anterior motion of the aortic root is decreased when

cardiac output is poor

Left atrium:
			Taken at maximal systolic excursion

Normal: ratio of LA: aortic root ~1:1; ratios >1.3 suggest LAE; imprecise measure

Systolic time intervals:
			Can be used to estimate LV function; influenced by cardiac filling and afterload

LVET (duration of time the aortic valve is open), preejection period (PEP; time from the onset of the QRS to aortic valve opening), total electromechanical systole (LVET plus PEP)

Reduced LV function leads to PEP prolongation and LVET shortening as SV decreases (increased PEP/LVET ratio)

Doppler Echocardiography

Echos from cells moving away from the transducer are of lower frequency and those from cells moving toward the transducer are of higher frequency

Doppler beam alignment- as long as the angle between the ultrasound beam and path of blood flow is less than 20 degrees, maximal flow velocity can be estimated with reasonable accuracy; as this angle increases, calculated velocity decreases

If perpendicular, the calculated velocity is 0

Flow signals are displayed with time on the horizontal axis and velocity (m/sec) on the vertical axis- zero baseline demarcates flow away from the transducer (below baseline) or toward (above baseline)

Doppler modes: Spectral Doppler (pulse wave (PW) and continuous wave (CW)) and color flow (CF) mapping

Pulsed wave: Uses short bursts of ultrasound to interrogate a specified area (sample volume)= range gating

Blood flow velocity, direction and spectral characteristics can be calculated from a specific location in the heart or blood vessel

Main disadvantage is that the max measureable velocity is limited by the transmitted frequency, the sample volume’s distance from the transducer and by the pulse repetition frequency (PRF)

PRF relates to the time required to send, receive and process echoes

Twice PRF=Nyquist limit defines max measureable velocity

Lower frequency transducers and closer sample volume placement increase the Nyquist limit

Blood velocity higher than the Nyquist limit cannot be measured because of signal aliasing

High pulse repetition frequency (HPRF) PW Doppler is a hybrid of PW and CW and allows sampling from several depths along the line of interrogation

Spectral broadening=widening of the time-velocity curve occurs with disparities in blood cell velocity and direction

Continuous wave: Uses dual crystals so that ultrasound can be continuously and

simultaneously transmitted and received

PRF is very high, theoretically no max velocity limit with CW

Disadvantage is that sampling occurs all along the line of interrogation= range

ambiguity

Color flow mapping:

Can show normal flow patterns and identify areas of abnormal flow

Most color code blood flow moving away from the transducer blue and flow

toward as red; zero velocity- black

Multiple velocities and directions of flow= turbulence are indicated using a variance map which adds shades of yellow or green

Signal aliasing occurs often because the Nyquist limits are usually quite low; signal aliasing is displayed as a reversal or wrap around of color

Many normal dogs- trivial to mild physiologic regurg of pulmonic valves

Many normal cats- physiologic tricuspid regurg; mitral/pulmonic uncommon

Color flow M-mode- use of CF mapping with M-mode imaging permits more precise

timing of flow events within the cardiac cycle

Done by placing the color sector over the area of interes in 2-D mode, then

aligning the M-mode cursor over this and switching to M-mode

Normal transvalvular flow:

Breed, age, weight- little influence on measurements

Flow patterns of AV valves are similar; mitral valve typically has higher velocities

compared with the tricuspid valve (peak E ~0.75m/sec, peak A ~0.55m/sec)

Flow patterns of semilunar valves are similar

The 4-chamber left apical view provides optimal alignment for imaging mitral inflow

Left cranial short-axis view allows good alignment with tricuspid inflow

Flow across the ventricular outflow tracts accelerates quickly during ejection with a

more gradual deceleration

Peak aortic velocity is usually 0.9-1.6 m or less; velocities >2.1-2.2m/sec are suggestive of outflow obstruction; 1.7-2.2 m/sec grey zone

Obtained from subcostal (subxiphoid) position; although some dogs may have higher velocities recorded from the left apical position

Peak pulmonary velocity is 1.4-1.5m/sec or less

Best assessed from left-sided positions

Modified Bernoulli equation

Pressure gradient=4 (maximum velocity)2

Peak velocity of a regurgitation jet can be used to estimate pressure in the

absence of stenosis

Left ventricular function

Cardiac output can be estimated from HR and the area under the ventricular outflow signal (proportional to stroke volume)

Doppler spectra with simultaneous ECG can be used to measure preejection period and

ejection time (STIs)

Aortic signal acceleration time correlates with LV systolic function but is load dependent

LV dP/dtmax can be estimated in patients with mitral regurg by the acceleration rate of

the regurgitant jet velocity signal

Doppler-derived indices of diastolic function: isovolumic relaxation time (IVRT),

characteristics of the mitral inflow pattern, pulmonary venous flow pattern, Doppler tissue imaging (DTI)

Transesophageal Echocardiography: Can provide clearer images of some cardiac structures compared with transthoracic echo

3D-Echocardiography