Design specifications

Still need to figure out how to regulate flow through one of the pipes and allow flow through the other.

Over Flow Weir

- Location:

  - Second Hopper

  - On side of walkway

  - Positioned parallel to Et

  - In slant of wall

- Dimensions:

  - Length of Full hopper

  - Same width/height as before

  - Stop short of trash rack to allow for the flow blocker

- Design:

  - Same as before, just longer weir

Flow Control Valve

Note: for the inlet coupling (where water is flowing into the flow control through the hopper) we will have an elbow coupling directed vertically to act as a pipe stub for the flow control valve on both sides. This solves the issue of creating a multipurpose flow control valve – you can regulate one flow or both of them now.

Note: this also solves the problem that arises when one Quarter hopper is full and the other is empty. Naturally by our flow control design water would flow in the individual inlet from the full tank, and while the valve is closed it would continue through to exit into the empty tank. This would have caused losses in flow rate at a huge proportion.

- Location:

  - Front of entrance tank (below inlet pipes)

  - Same height as past design

  - Two pipes, each on either side of the removable wall.

- Dimensions:

  - Same size as current outflow pipe.

- Design:

  - Pipe out the front of tank

  - Elbow joint to turn pipes parallel to Et front wall, directly at each other

  - T-Coupling to adjoin two pipes, Direct adjoined pipe down

  - Pipe vertical, with Valve coupling on it, down to drain

  - Elbow joint to direct vertical flow horizontally under the Et into the drain channel

- Concerns:

  - Interior hopper elbow coupling solves main issues

Pipe Stub / Hopper Stopper

- Location:

  - On either side of cement wall, on flat ground in quarter hoppers

  - Close to wall, to reach drain channel

- Dimensions:

  - Same as current Pipe stub valves

- Concerns:

  - Make sure they have room to both hit the drain channel underneath

Removable Wall / partition

- Location:

  - Parallel to Et flow

  - Spanning from front Et wall to trash rack

  - Wall supports along slope, wall and ground to hold in place

- Dimensions:

  - Width TBT

  - Length = Length of half hopper

  - Width of supports TBD

- Design:

  - Mica – light material good for this.

  - Wall needs to be removable but not often

- Concerns:

  - Need to make sure cement supports are constructible.

Removable trash rack overflow wall

Note: this design is necessary to prevent water from flowing from the full hopper back into the first quarter hoppers in the event that one inlet is being discarded and that hopper is empty.

- Location:

  - On ledge between first and second hopper.

  - In front of trash rack

  - Spanning from cement wall in center of hopper to one wall on the side of the Et.

- Dimensions:

  - Similar thickness to removable wall

  - Same thickness of cement supports as removable wall

- Design:

  - This removable wall needs to block off flow from full hopper back into one of the quarter hoppers. To do so it must join up with cement wall fairly well.

  - Must be removable as well, adjusted based off which inlet is being wasted.

  - Adjoin with T cement coupling

- Concerns:

  - Going to be annoying to create more cement supports and get another removable wall.

Trash Rack

- Location:

  - Moved back to allow for the removable partition