MAE 5020 Wind Blade Fluent Steps

Geometry:

- Import STL of wind blade into Geometry

- Open SpaceClaim

- Check units – set to mm, etc.

- Rotate wind blade such that fluid flow direction is along the positive Z-axis direction, blade axis along Y-direction.

- Select tip face, hide all other faces

- Click z-axis, create new plane (hit Escape so it stops creating more planes)

- View from y-axis

- Create points at leading and trailing edges (carefully) and draw the chord line, it becomes part of the

- Go back to 3D mode – face is split but this is OK – hide points, highlight the plane, measure angle between xy-plane and chord line

- Zoom extents  show all faces, select body, move, anchor origin, rotate about y-axis to get it to neutral.

- Then rotate it to get the pitch setting right

- Delete plane and line to keep a single face

- Move the wind blade 40mm away from the origin in the y-direction to simulate the hub of the wind blade.

BREAK -

- Use XY plane as a sketch plane, create a 1/3 circle with the reference frame as the center of the circle. The wind blade should be oriented along the x-axis and the edges of the cylinder are 60 degrees from the x-axis in both directions. The circle radius should be 2.5 wind blade lengths.

- Using pull command, create a solid 1/3 cylinder by extruding the surface 2.5 wind blade lengths for the inlet and 5 wind blade lengths for the outlet. The blade used for the 4021/5020 project has a length of about 300 mm when including the hub.

  - Select “no merge” to make sure not to merge the wind blade into the extrusion

- Use the combine function, select the enclosure as the desired shape and the wind blade as the cutter body. Deselect “Keep Cutter”

- 3 solids should be in the Structure tree; the wind blade, and the enclosure with the wind blade. Suppress wind blade and hide it.

  - Verify results by hiding a face of the enclosure and confirming that the wind blade is still in the enclosure

- Create the following named selections: inlet, inlet_top, outlet, Period 1, Period 2

  - Periods should be defined so that the blade rotating around positive z has P1 leading

  - Rename the enclosure under structures as fluid_domain

  - Highlight the wind blade body and create a named selection called blade

- SAVE

  - Include external files if saving as wbpz

Fluent Mesher:

- Drag the Fluent with Fluent Meshing tool from the ANSYS toolbox into the ANSYS workspace

- Drag the geometry from the Geometry tool into the Mesh option Fluent Meshing

- Launch the Fluent Mesher, enabling double precision and selecting the desired number of cores

- Display  Mouse Buttons   workbench default.

- Import geometry

- To use clipping planes: Display all  insert clipping planes  limits in z, y, x as desired

- Add Local Sizing to the blade named selection, use defaults, it shows little sample cells

- Use defaults, create surface mesh

  - Curvature & Proximity turned on

  - CHECK TRAILING EDGE using clipping planes

- Use fluid region with no voids under Describe Geometry, select no and no for next two boxes

- Right click on Describe Geometry, select new task, then select set up periodic boundaries

  - Use default settings (Rotational) and select Period 1 and 2

  - Check the trailing edge – if it is no longer correct, change “Remesh Asymmetric Mesh Boundaries” from “auto” to “no”

- Should by default pick the right boundaries

- Click Update Regions

  - Should be two regions, blade and fluid domain

  - Set the blade region to dead (should be fluid by default) (fluent reads it just as a cavity)

  - Leave fluid_domain as fluid

- Add a boundary layer using default values

- Create Volume mesh using default values – check the units to match what I was using in Spaceclaim

- Once mesh is complete, check the number of nodes/cells using report and check mesh under the mesh tab

  - Check curvature and proximity on surface mesh if we are having overlapping errors

- SAVE

- Check mesh and click Switch to Solution if Mesh is correct

Model Setup

- Under Setup, Models, select Viscous

  - Pick k-omega for model and GEKO for k-omega model

  - For this test, use default air values (Materials -> Fluid)

  - Should be 1.225 for density and 1.7894e-5 for viscosity

- Under Cell Zone Conditions > Fluid >  fluid_domain

  - Select frame motion, set Rotation-axis origin to {0,0,0} and rotation-axis direction to {0,0,1}. Set rotation velocity to 98 rad/s (tip speed ratio of about 5)

- Under boundary conditions

  - Select inlet, change magnitudes to components and set the Z-velocity component to 6 m/s

  - Repeat 26a for inlet_top

  - For outlet, ensure the gauge pressure is 0 and the operating conditions is 101325 Pa

  - Check that the periodic boundary conditions are set.

Numerical Solution:

- Under Solutions (top bar), select definitions, new, surface, integral

  - Ensure field variables are set to pressure, static pressure

  - Select surface to be blade

  - Under Create, select Report File and Report Plot

- Expand Monitors, double click on Residual

  - Keep absolute criteria at 1e-3

- Under Initialization, select Standard Initialization

  - Compute From > inlet, double check values (Z-velocity should be 6 m/s)

  - Click Initialize

- Click on Run Calculation, set number of iterations, then run calculation

Postprocessing:

- Pressure contour on the blade

  - Countour  name  pressure  global  100 contours

- Pressure contour & Velocity Vectors on section y  = .1

  - Location  create plane  xz  plane  pick y = .1

  - Contour  location plane 1  variable pressure  range local  100 contours

- Power

  - Calculators  location blade wall  axis global Z  fluid air  calculate

  - Multiply by angular speed to get power output.