ECHOCARDIOGRAPHY CHAPTER 11

- Gives info about cardiac structure, dimensions, and function

- Different types of echocardiography

  - M-mode

  - 2D

  - 3D

  - Doppler echocardiography – spectral, tissue Doppler imaging

- M-mode

  - Depicts linear dimension plotted against time

  - ONE PLANE only

  - +: excellent time resolution for measurement of cardiac dimensions at precise time points

  - - : sampling area is very limited; gives little information about changes in cardiac size or shape in other dimensions, or about specific lesions; difficult technique

  - Can be used for:

    - LA:AoM-mode

    - M-mode LV diameter

- 2D

  - Sector scan that produces tomographic slices through the heart

  - Provides numerous different imaging planes

  - Can show abnormalities of cardiac size, shape, lesions.

  - Different views:

    - Right parasternal long-axis (4 chamber and 5 chamber)

    - Right parasternal short axis (papillary muscle level, mitral valve level, aortic valve level, pulmonary artery level)

    - Left apical (4 chamber, 5 chamber, 2 chamber)

    - Left cranial (tricuspid valve, aorta, pulmonary artery)

- Doppler echocardiography

  - ** Doppler principle: frequency of reflected US waves is changed if the reflecting target is moving towards or away from the source

  - **The beam of US waves MUST be parallel with the direction of the moving target (i.e. blood flow)

  - Gives information about velocity of BF (spectral)

    - Velocity displayed as a velocity-time graph

    - Blood flow away from transducer: below the baseline

    - Blood flow towards the transducer: above the baseline

    - Pulsed wave (PW): very limited area of BF is sampled

    - Continuous wave (CW): BF is sampled along the entire length of the cursor; higher velocities can be displayed; range ambiguity: the exact location of the high velocity flow along the cursor cannot be determined from the spectral signal

    - ** Simplified Bernoulli equation:

Pressure gradient (mmHg) = 4 x velocity2

  - Direction and timing of BF can be depicted in color (color Doppler)

    - Blue: away ; Red: towards

    - Aliasing: high velocities; reversal of color (red to blue. blue to red)

  - Myocardium velocity can also be recorded (tissue Doppler imaging)

Chambers Measurements

- * Left heart – partly because the left heart is an easier shape to quantify than the right; most important acquired diseases primarily affect the left heart

- Measurements should be taken from at least 3 cardiac cycles and averaged

- LA:

  - M-mode or 2D

- LV

  - M-mode or 2D

  - Normal canine RI are problematic because of the range in BW and conformation – ratio indices, Cornell method, breed-specific values

Systolic Function

- Fractional shortening = the percentage change in LV diameter in systole

  - M-mode obtained from short-axis

  - Timing:

    - LVDd measured at end-diastole at the start of QRS

    - LVDs measured at peak septal motion

  - FS = (LVDd-LVDs)/LVDd x100 ; cats: 30-50%; dogs: 25-50%

- Mitral E-point to septal separation (EPSS)

  - M-mode

  - To measure maximal opening of the mitral valve in diastole by measuring the distance between the septum and early opening (E-point) of the anterior mitral valve leaflet (EPSS)

  - This distance is increased with systolic dysfunction; large dogs: < 7 mm

  - !! Avoid oblique view

  - Timing:

    - Peak opening of mitral valve in early systole

- Left ventricle end-systolic volume index

  - ESV can be normalized to BSA; ESVI = ESV/BSA ≤ 30 ml/m2

  - Timing: ESV at smallest LV dimensions before mitral valve opens

  - Trace endocardial border from septal mitral valve annulus to lateral wall mitral valve annulus

  - Vital to obtain true apex

- Ejection fraction

  - The percentage change in LV volume during systole can be calculated from the volumes measured from a right parasternal long-axis view

  - Should not be derived from volumes estimated from M-mode measurements

  - When calculated from 2D images, EF more accurately assesses global systolic function

  - Timing:

    - EDV after mitral valve closes before systole

    - ESV: at smallest LV dimensions before mitral valve opens

  - EF% = (EDV- ESV)/EDV x 100;  dogs = 50-65%

- Systolic time intervals

  - Uses the timing of ejection with respect to electrical activation

  - The interval between the beginning of the Q and the onset of ejection of blood into the aorta (the pre-ejection period; PEP) can be divided by the duration of aortic ejection (the ejection time; ET) to give the systolic time interval = PEP/ET

  - This ratio is increased with systolic dysfunction and can be measured from the spectral Doppler flow signal with a concurrent ECG; dogs = 0.24-0.38

  - Timing:

    - PEP: from start of QRS to onset of aortic ejection

    - ET: duration of aortic spectral Doppler flow signal

Questions

- You are evaluating a 5 year old male neutered Labrador. You obtained the following echocardiographic values: LVDd = 4.7 cm; LVDs = 3.1 cm; IVSd = 1.0 cm; LVFWd = 1.0 cm; LA = 2.7 cm; Ao=2/6 cm/

What is his fractional shortening? Is that normal? Which view do you use to evaluate FS?

- When do you measure the LVDs?

  - At the onset of the QRS

  - At the end of the QRS

  - At the onset of the T wave

  - At the peak of the T wave

- True or False: In spectral Doppler, if the beam is not align perfectly parallel with blood flow, the peak velocity will be overestimated.

Questions & Answers

- You are evaluating a 5 year old male neutered Labrador. You obtained the following echocardiographic values: LVDd = 4.7 cm; LVDs = 3.1 cm. What is his fractional shortening? Is that normal?

FS = (4.7-3.1)/4.7 x 100 = 34%; normal; Short-axis view, right parasternal window (midventricular region)  M-mode

- When do you measure the LVDs?

  - At the onset of the QRS - diastole

  - At the end of the QRS

  - At the onset of the T wave

  - At the peak of the T wave

- False: In spectral Doppler, if the beam is not align perfectly parallel with blood flow, the peak velocity will be underestimated